- Limitando acesso

  - Cisco IOS Command-Based Switch

| enable password level 1 password | Exec usuário |
| --- | --- |
| enable password level 15 password | Exec previlegiado |

  - Cisco Set Command-Based Switch

| set password | Exec usuário |
| --- | --- |
| set enablepass | Exec previlegiado |

- Visualizando a configuração

  - Cisco IOS Command-Based Switch

| show running-config |  |
| --- | --- |
| delete nvram |  |
| copy running-config startup-config |  |

  - Cisco Set Command-Based Switch

| show config |  |
| --- | --- |
| show ip |  |
| clear config all |  |

- Identificação do switch

  - Cisco IOS Command-Based Switch

| hostname name |  |
| --- | --- |

  - Cisco Set Command-Based Switch

| set prompt name |  |
| --- | --- |

- Configuração IP

  - Cisco IOS Command-Based Switch

| ip address endereço_IP mascara | A configuração no 1900/2820 é feita no modo global, enquanto que nos outros é feita na configuração de vlan’s (a vlan 1 é, por default, associada a rede de gerenciamento. |
| --- | --- |

  - Cisco Set Command-Based Switch

| set interface sc0 endereço_IP mascara |  |
| --- | --- |

- Configuração das interfaces

  - Cisco IOS Command-Based Switch

| speed 10/100/auto |  |
| --- | --- |
| duplex half/full/auto |  |
| description descrição |  |
| show interface type mod/porta |  |

  - Cisco Set Command-Based Switch

| set port speed mod/porta 10/100/auto |  |
| --- | --- |
| set port duplex mod/porta full/half/auto |  |
| set port name mod/porta descrição |  |
| show port mod/porta |  |

- Gerência de Vlans

  - Cisco IOS Command-Based Switch

| vlan vlan_id name name | Criando vlans no modo de configuração global (1900/2820/6000) |
| --- | --- |
| vlan-membership static vlan_ID | Modo de configuração de interface (1900/2820/6000) |
| vlan-membership dynamic | Modo de configuração de interface (1900/2820/6000) |
| vlan database vlan vlan_ID | (6000) |
| show vlan nome_vlan | (6000) |
| interface vlan vlan_ID | Selecão da vlan no modo de configuração (2900/2950) |
| switchport access vlan_ID | (2900/2950) |
| switchport access dynamic | (2900/2950) |
| switchport mode access/multivlan/trunk/ | (2900/2950) |
| show interface mod/porta switchport |  |
| show vlan |  |
| show vlan name nome_da_vlan |  |
| show vlan id vlan_ID |  |

  - Cisco Set Command-Based Switch

| show vlan |  |
| --- | --- |
| set vlan vlan_id mod/lista_de_portas |  |

- Gerenciando  Vlan Trunks

  - Cisco IOS Command-Based Switch

| trunk on/off/negociate/auto/nonegociate | (1900/2820/6000) |
| --- | --- |
| switchport mode trunk | Configuração de interface |
| switchport trunk vlan allowed faixa_de_vlans |  |
| switchport trunk encapsulation dot1q/isl |  |
| switchport trunk pruning faixa_de_vlans |  |
| show interface mod/porta switchport |  |

  - Cisco Set Command-Based Switch

| set trunk mod/porta on/off/desirable/auto/negociate vlan_range isl/dot1q¦lane¦negotiate |  |
| --- | --- |
| show trunk mod/porta |  |

- Gerenciando VTP (VLAN Trunk Protocol)

  - Cisco IOS Command-Based Switch

| vtp domain domínio | (6000 IOS Native) |
| --- | --- |
| vtp mode client/server/transparent | (6000 IOS Native) |
| vlan database | (2900/2950/3500) – O comando deve ser usado no modo exec privilegiado |
| vtp server vtp client vtp transparent | (2900/2950/3500) |
| vtp domain nome_do_domínio |  |
| vtp v2-mode |  |
| vtp password senha |  |
| show vtp status |  |

  - Cisco Set Command-Based Switch

| set vtp v2 enable |  |
| --- | --- |
| set vtp domain nome passwd senha |  |
| set vtp domain nome mode server/client/transparent |  |
| set vtp ptruneeligible faixa_de_vlans |  |
| show vtp domínio |  |

- Gerenciando Spanning Tree

  - Cisco IOS Command-Based Switch

| show spanning-tree [vlan <vlan_ID>] |  |
| --- | --- |

  - Cisco Set Command-Based Switch

- Gerenciando Port Aggregation

  - Cisco IOS Command-Based Switch

| channel-Group grupo mode on/desirable/auto |  |
| --- | --- |
| show int etherchannel |  |

  - Cisco Set Command-Based Switch

Recuperação de senha de switch (2900, 2950XL, 3500)

Step 1 Connect a terminal or PC with terminal emulation software to the console port.

For more information, refer to the switch installation guide.

Step 2 Set the line speed on the emulation software to 9600 baud.

Step 3 Unplug the switch power cord.

Step 4 Press in the Mode button, and at the same time reconnect the power cord to the switch.

You can release the Mode button a second or two after the LED above port 1X goes off. Several lines of information about the software appear, as do instructions:

The system has been interrupted prior to initializing the flash file system. The following commands will initialize the flash file system, and finish loading the operating system software:

flash_init

boot

Step 5 Initialize the Flash file system:

switch: flash_init

Step 6 If you had set the console port speed to anything other than 9600, it has been reset

to that particular speed. Change the emulation software line speed to match that

of the switch console port.

Step 7 Display the contents of Flash memory as in this example:

switch: dir flash:

The switch file system is displayed:

Directory of flash:/

3 drwx     10176 Mar 01 2001 00:04:34 html

6 -rwx      2343 Mar 01 2001 03:18:16 config.text

171 -rwx   1667997 Mar 01 2001 00:02:39 c2950-c3h2s-mz.120-5.WC.1.bin

7 -rwx 3060 Mar 01 2001 00:14:20 vlan.dat

172 -rwx 100 Mar 01 2001 00:02:54 env_vars

7741440 bytes total (4788224 bytes free)

Step 8 Rename the configuration file to config.text.old.

This file contains the password definition.

switch: rename flash:config.text flash:config.text.old

Step 9 Boot the system:

switch: boot

You are prompted to start the setup program. Enter N at the prompt:

Continue with the configuration dialog? [yes/no]: N

Step 10 At the switch prompt, change to privileged EXEC mode:

switch> enable

Step 11 Rename the configuration file to its original name:

switch# rename flash:config.text.old flash:config.text

Step 12 Copy the configuration file into memory:

switch# copy flash:config.text system:running-config

Source filename [config.text]?

Destination filename [running-config]?

Press Return in response to the confirmation prompts.

The configuration file is now reloaded, and you can use the following normal

commands to change the password.

Step 13 Enter global configuration mode:

switch# config terminal

Step 14 Change the password:

switch(config)# enable secret <password>

or

switch(config)# enable password <password>

Step 15 Return to privileged EXEC mode:

switch(config)# exit

switch#

Step 16 Write the running configuration to the startup configuration file:

switch# copy running-config startup-config

The new password is now included in the startup configuration.