Prova prática semestre III_V1

- Configuração básica IGRP grupo de 3 roteadores. 					 20 pontos

Itens avaliados = 15

Router igrp 200, 2 pontos por roteador

Network, 1 ponto por rede

| INT | Lab_A | Lab_B | Lab_C | Lab_D | Lab_E | Lab_F | Lab_G | Lab_H | Lab_I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0 | 201.230.124.0 | 201.10.11.0 | 199.6.13.0 | --------------- | 205.56.213.0 | 219.27.28.0 | 207.17.50.0 | 210.93.105.0 | 204.204.7.0 |
| S1 | 204.204.7.0 | 201.230.124.0 | 201.10.11.0 | 199.6.13.0 | ----------------- | 205.56.213.0 | 219.27.28.0 | 207.17.50.0 | 210.93.105.0 |
| E0 | 220.205.75.0 | 205.7.5.0 | 192.5.5.0 | 219.17.100.0 | 195.109.140.0 | 221.157.189.0 | 210.78.212.0 | 219.54.60.0 | 199.64.3.0 |
| E1 | --------------------- | -------------------- | ----------------- | 223.8.151.0 | 223.8.151.0 | ----------------- | --------------- | 223.8.151.0 | ------------------ |

Passive interface E0, 1 ponto por roteador

Nos 3 roteadores depende do grupo 18 / 19 pontos, neste caso o aluno leva o bônus e ganha 20 pontos.

- Roteamento IGRP ?	 2 justificativas							6 pontos

2 caminhos, 2 pontos

Verificando o tabela de roteamento existem duas entradas com a mesma métrica, 2 pontos

Usando o comando traceroute verificamos que os pacotes seguem por dois caminhos : Lab_D, C, B, e Lab_H, I, A ,2 pontos

- Roteamento IGRP ?										6 pontos

Variance

  - Todos, 2 pontos

  - Variance multiplier + maximum-paths default, 2 pontos

  - maximum-paths 2, 2 pontos

- Configuração IPX.										21 pontos

Itens avaliados = 12

Interface E1.1 network 1A    encapsulation sap

Interface E1.1 network 1B    encapsulation novell-ether

Interface E1.1 network 1C    encapsulation arpa

| INT | Lab_A | Lab_B | Lab_C | Lab_D | Lab_E | Lab_F | Lab_G | Lab_H | Lab_I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0 | D1 | D5 | D6 | --------------- | D7 | D8 | D4 | D3 | D2 |
| S1 | D2 | D1 | D5 | D6 | --------------- | D7 | D8 | D4 | D3 |
| E0 | 1E | 4E | 5E | 6E | 7E | 8E | 9E | 2E | 3E |
| E1 | --------------------- | --------------------- | ----------------- | 1A,1B,1C | 1A,1B,1C | --------------- | --------------- | 1A,1B,1C | ------------------ |

IPX routing = 1 ponto

WAN = 1 ponto, 5 ou 6 WAN por grupo, todas = 6 pontos

E0 = 2 pontos

E1 = 2 pontos por cápsula.

- Listas de acesso.										27 pontos

Itens avaliados = 32

Cada linha =  0,5 ponto

Aplicar na interface  =  4 pontos

Aplicar na line  =  3 pontos

Grupo ( AIH ) , ( BCD ) , ( EFG )

Access-list 101 permit ip host 223.8.151.4 any			Host instrutor

Access-list 101 permit icmp host 205.7.5.1 any  			E0 Lab_B

Access-list 101 permit icmp host 192.5.5.1 any  			E0 Lab_C

Access-list 101 permit icmp host 219.17.100.1 any  			E0 Lab_D

Access-list 101 permit icmp host 195.109.140.1 any  			E0 Lab_E

Access-list 101 permit icmp host 221.157.189.1 any  			E0 Lab_F

Access-list 101 permit icmp host 210.78.212.1 any  			E0 Lab_G

Access-list 101 permit icmp host 219.54.60.1 any  			E0 Lab_H

Access-list 101 permit icmp host 199.64.3.1 any  			E0 Lab_I

Access-list 101 permit icmp host 220.205.75.1 any  			E0 Lab_A

Access-list 101 permit icmp 201.230.124.0 0.0.0.3 any  		Wan Lab_A-B

Access-list 101 permit icmp 201.10.11.0 0.0.0.3 any  			Wan Lab_B-C

Access-list 101 permit icmp 199.6.13.0 0.0.0.3 any 			Wan Lab_C-D

Access-list 101 permit icmp 205.56.213.0 0.0.0.3 any  			Wan Lab_E-F

Access-list 101 permit icmp 219.27.38.0 0.0.0.3 any  			Wan Lab_F-G

Access-list 101 permit icmp 207.17.50.0 0.0.0.3 any 			Wan Lab_G-H

Access-list 101 permit icmp 210.93.105.0 0.0.0.3 any  			Wan Lab_H-I

Access-list 101 permit icmp 204.204.7.0 0.0.0.3  any  			Wan Lab_I-A

Access-list 101 permit icmp 223.8.151.0 0.0.0.3 any  			Ethernet Lab_D-E-H

Access-list 101 deny icmp any any

Access-list 101 permit ip any any

Lab_a int s0 ip access-group 101 in , Lab_h int s1 ip access-group 101 in , Lab_h int e1 ip access-group 101 in

Lab_b int s1 ip access-group 101 in , Lab_d int e1 ip access-group 101 in

Lab_g int s0 ip access-group 101 in , Lab_e int e1 ip access-group 101 in

Acesso telnet grupo ( EFG )

Access-list 2 deny  host 205.7.5.1

Access-list 2 deny  host 192.5.5.1

Access-list 2 deny  host 219.17.100.1

Access-list 2 permit  205.7.5.0 0.0.0.255

Access-list 2 permit  192.5.5.0 0.0.0.255

Access-list 2 permit  219.17.100.0 0.0.0.255

Access-list 2 permit  195.109.140.0 0.0.0.255

Access-list 2 permit  221.157.189.0 0.0.0.255

Access-list 2 permit  210.78.212.0 0.0.0.255

Access-list 2 permit  205.56.213.0 0.0.0.3

Access-list 2 permit  219.27.38.0 0.0.0.3

Access-list 2 permit  host 223.8.151.4

Line vty 0 4 ip access-class 2 in

Acesso telnet grupo ( AIH )

Access-list 2 deny  host 105.109.140.1

Access-list 2 deny  host 221.157.189.1

Access-list 2 deny  host 210.78.212.1

Access-list 2 permit  195.109.140.0 0.0.0.255

Access-list 2 permit  221.157.189.0 0.0.0.255

Access-list 2 permit  210.78.212.0 0.0.0.255

Access-list 2 permit  220.205.75.0 0.0.0.255

Access-list 2 permit  199.64.3.0 0.0.0.255

Access-list 2 permit  219.54.60.0 0.0.0.255

Access-list 2 permit  210.93.105.0 0.0.0.3

Access-list 2 permit  204.204.7.0 0.0.0.3

Access-list 2 permit  host 223.8.151.4

Line vty 0 4 ip access-class 2 in

Acesso telnet grupo ( BCD )

Access-list 2 deny  host 220.205.75.1

Access-list 2 deny  host 199.64.3.1

Access-list 2 deny  host 219.54.60.1

Access-list 2 permit  220.205.75.0 0.0.0.255

Access-list 2 permit  199.64.3.0 0.0.0.255

Access-list 2 permit  219.54.60.0 0.0.0.255

Access-list 2 permit  205.7.5.0 0.0.0.255

Access-list 2 permit  192.5.5.0 0.0.0.255

Access-list 2 permit  219.17.100.0 0.0.0.255

Access-list 2 permit  201.10.11.0 0.0.0.3

Access-list 2 permit  199.6.13.0 0.0.0.3

Access-list 2 permit  host 223.8.151.4

Line vty 0 4 ip access-class 2 in

- Configuração de Switch / vlan’s + spanning tree protocol.				20 pontos

Itens avaliados = 33

Hostname =  1 ponto

IP Vlan1   = 2 pontos

Enable secret  cisco= 1 ponto

VTP mode client = 1 ponto

VTP domain CCNA = 1 ponto

VTP password class = 2 ponto

Interface fastethernet 0/26 trunk = 2 pontos

Interface fastethernet 0/27 trunk = 2 pontos

Vlan’s = 0,1 ponto

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 26 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 1 | T | T |
| C | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 2 | 2 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 1 | T | T |
| D | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 3 | 3 | 2 | 2 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 1 | T | T |
| E | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 3 | 4 | 4 | 2 | 2 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 1 | T | T |
| F | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 3 | 3 | 4 | 4 | 5 | 5 | 2 | 2 | 7 | 7 | 8 | 8 | 9 | 9 | 1 | T | T |
| G | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 2 | 2 | 8 | 8 | 9 | 9 | 1 | T | T |
| H | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 7 | 2 | 2 | 9 | 9 | 1 | T | T |
| I | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 2 | 2 | 1 | T | T |

STP = 5,6 pontos