Exercicio Frame Relay

Parte 1 (PVCs em redes diferentes)

Roteador Lab_E (funcionando como folha na topologia do laboratório)

Current configuration:

!

version 12.0

service timestamps debug uptime

service timestamps log uptime

service password-encryption

!

hostname Lab_e

!

enable secret 5 $1$V8UJ$qr5S0s.6gyxIpKFblwsvc1

!

ip subnet-zero

ip host Lab_a 192.5.5.1 205.7.5.1 201.100.11.1

ip host Lab_b 201.100.11.2 219.17.100.1 199.6.13.1

ip host Lab_c 199.6.13.2 223.8.151.1 204.204.7.1

ip host Lab_e 210.93.105.2

ip host Lab_d 204.204.7.2 210.93.105.1

!

!

!

interface Ethernet0

ip address 210.93.105.2 255.255.255.0

no ip directed-broadcast

!

interface Serial0

no ip address

no ip directed-broadcast

encapsulation frame-relay

!

interface Serial0.1 point-to-point

description ###################### NAO MEXA AQUI MINEIRO!!! ####

ip address 199.6.13.2 255.255.255.0

no ip directed-broadcast

no arp frame-relay

frame-relay interface-dlci 200

!

interface Serial0.2 point-to-point

ip address 204.204.7.2 255.255.255.0

no ip directed-broadcast

frame-relay interface-dlci 310

!

interface Serial1

no ip address

no ip directed-broadcast

shutdown

!

router rip

network 210.93.105.0

!

ip nat translation timeout never

ip nat translation tcp-timeout never

ip nat translation udp-timeout never

ip nat translation finrst-timeout never

ip nat translation syn-timeout never

ip nat translation dns-timeout never

ip nat translation icmp-timeout never

no ip classless

!

!

line con 0

password 7 094F471A1A0A

logging synchronous

login

transport input none

line aux 0

password 7 094F471A1A0A

logging synchronous

login

line vty 0 4

password 7 094F471A1A0A

login

!

end

Roteador Lab_A (funcionando como frame-relay switching)

Current configuration:

!

version 11.1

service password-encryption

service udp-small-servers

service tcp-small-servers

!

hostname Lab_a

!

enable secret 5 $1$SG0s$fDjDq8KbIzxRan4NpxlnP1

enable password 7 104D000A0618

!

no ip domain-lookup

frame-relay switching

!

interface Ethernet0

ip address 192.5.5.1 255.255.255.0

media-type 10BaseT

!

interface Ethernet1

ip address 205.7.5.1 255.255.255.0

media-type 10BaseT

!

interface Serial0

no ip address

no ip directed-broadcast

encapsulation frame-relay

no fair-queue

frame-relay intf-type dce

frame-relay route 100 interface Serial1 150

frame-relay route 410 interface Serial2 400

!

interface Serial1

no ip address

encapsulation frame-relay

frame-relay intf-type dce

frame-relay route 150 interface Serial0 100

frame-relay route 200 interface Serial3 310

!

interface Serial1.1 point-to-point

ip address 10.10.0.1 255.0.0.0

frame-relay interface-dlci 300

!

interface Serial2

no ip address

encapsulation frame-relay

frame-relay intf-type dce

frame-relay route 400 interface Serial0 410

frame-relay route 410 interface Serial3 200

!

interface Serial3

no ip address

encapsulation frame-relay

clockrate 2000000

frame-relay intf-type dce

frame-relay route 200 interface Serial2 410

frame-relay route 310 interface Serial1 200

!

router rip

network 201.100.11.0

network 192.5.5.0

network 205.7.5.0

network 10.0.0.0

!

no ip classless

!

line con 0

password 7 060506324F41

login

line aux 0

password 7 104D000A0618

login

line vty 0 4

exec-timeout 30 0

password 7 105E0110091E0218

login

!

end

Parte 2 (Redes iguais em todos os PVCs) - topologia Hub-and-Spoke

Roteador Lab_E (folha)

Current configuration:

!

version 12.0

service timestamps debug uptime

service timestamps log uptime

service password-encryption

!

hostname Lab_e

!

enable secret 5 $1$V8UJ$qr5S0s.6gyxIpKFblwsvc1

!

ip subnet-zero

ip host Lab_a 192.5.5.1 205.7.5.1 201.100.11.1

ip host Lab_b 201.100.11.2 219.17.100.1 199.6.13.1

ip host Lab_c 199.6.13.2 223.8.151.1 204.204.7.1

ip host Lab_e 210.93.105.2

ip host Lab_d 204.204.7.2 210.93.105.1

!

!

!

interface Ethernet0

ip address 210.93.105.2 255.255.255.0

no ip directed-broadcast

!

interface Serial0

ip address 172.16.0.3 255.255.255.0

no ip directed-broadcast

encapsulation frame-relay

frame-relay map ip 172.16.0.4 310

frame-relay map ip 172.16.0.2 310

frame-relay local-dlci 310

!

interface Serial1

no ip address

no ip directed-broadcast

shutdown

!

ip nat translation timeout never

ip nat translation tcp-timeout never

ip nat translation udp-timeout never

ip nat translation finrst-timeout never

ip nat translation syn-timeout never

ip nat translation dns-timeout never

ip nat translation icmp-timeout never

no ip classless

!

!

line con 0

password 7 094F471A1A0A

logging synchronous

login

transport input none

line aux 0

password 7 094F471A1A0A

logging synchronous

login

line vty 0 4

password 7 094F471A1A0A

login

!

end

Roteador Lab_B (Roteador folha que centraliza o Hub-and-Spoke)

Building configuration...

Current configuration:

!

version 12.0

service timestamps debug uptime

service timestamps log uptime

service password-encryption

!

hostname lab_b

!

enable password 7 01100F175804

!

username cisco password 7 094F471A1A0A

username mmaia password 7 77810216

username lab_c password 7 00071A150754

ip subnet-zero

no ip domain-lookup

ip host LAB_A 201.100.11.1 192.5.5.1 205.7.5.1

ip host LAB_B 201.100.11.2 219.17.100.1 199.6.13.1

ip host LAB_C 204.204.7.1 199.6.13.2 223.8.151.1

ip host LAB_D 204.204.7.2 210.93.105.1

ip host LAB_E 210.93.105.2

!

!

!

interface Ethernet0

description CONECTADO AO HUB

ip address 219.17.100.1 255.255.255.0

no ip directed-broadcast

!

interface Ethernet0.1

no ip directed-broadcast

!

interface Ethernet0.2

no ip directed-broadcast

!

interface Ethernet0.3

no ip directed-broadcast

!

interface Serial0

description CONECTADO AO LAB_C S1

no ip address

no ip directed-broadcast

encapsulation frame-relay

!

interface Serial0.600 multipoint

ip address 172.16.0.1 255.255.0.0

no ip directed-broadcast

frame-relay interface-dlci 100

frame-relay interface-dlci 200

frame-relay interface-dlci 300

!

interface Serial1

description CONECTADO AO LAB_A S0

bandwidth 64

no ip address

no ip directed-broadcast

!

ip nat translation timeout never

ip nat translation tcp-timeout never

ip nat translation udp-timeout never

ip nat translation finrst-timeout never

ip nat translation syn-timeout never

ip nat translation dns-timeout never

ip nat translation icmp-timeout never

ip classless

ip route 193.22.5.0 255.255.255.0 199.6.13.2

ip route 199.198.1.0 255.255.255.0 199.6.13.2

!

access-list 100 deny   tcp host 219.17.100.1 host 219.17.100.5 eq telnet

access-list 100 deny   tcp host 219.17.10.4 host 223.8.151.2 eq echo

access-list 100 deny   tcp host 219.17.100.4 host 223.8.151.2 eq echo

access-list 101 permit tcp any eq telnet any

banner motd ^C

VOCE ESTA CONECTADO AO LAB_B ^C

!

line con 0

exec-timeout 0 0

password 7 045802150C2E

logging synchronous

login

transport input none

line aux 0

session-timeout 300

password 7 045802150C2E

absolute-timeout 200

logging synchronous

login

line vty 0 4

exec-timeout 200 0

password 7 045802150C2E

logging synchronous

login

!

end

Roteador Lab_A (Atua como switch frame-relay)

Current configuration:

!

version 11.1

service password-encryption

service udp-small-servers

service tcp-small-servers

!

hostname Lab_a

!

enable secret 5 $1$SG0s$fDjDq8KbIzxRan4NpxlnP1

enable password 7 104D000A0618

!

no ip domain-lookup

frame-relay switching

!

interface Ethernet0

ip address 192.5.5.1 255.255.255.0

media-type 10BaseT

!

interface Ethernet1

ip address 205.7.5.1 255.255.255.0

media-type 10BaseT

!

interface Serial0

no ip address

no ip directed-broadcast

encapsulation frame-relay

no fair-queue

frame-relay intf-type dce

frame-relay route 100 interface Serial1 150

frame-relay route 200 interface Serial3 310

frame-relay route 300 interface Serial2 400

!

interface Serial1

no ip address

encapsulation frame-relay

frame-relay intf-type dce

frame-relay route 150 interface Serial0 100

!

interface Serial2

no ip address

encapsulation frame-relay

frame-relay intf-type dce

frame-relay route 400 interface Serial0 300

!

interface Serial3

no ip address

encapsulation frame-relay

clockrate 2000000

frame-relay intf-type dce

frame-relay route 310 interface Serial0 200

!

router rip

network 201.100.11.0

network 192.5.5.0

network 205.7.5.0

network 10.0.0.0

network 172.16.0.0

!

no ip classless

ip route 10.0.0.0 255.0.0.0 10.10.0.2

!

line con 0

password 7 060506324F41

login

line aux 0

password 7 104D000A0618

login

line vty 0 4

exec-timeout 30 0

password 7 105E0110091E0218

login

!

end